UNIVERSITÉ DE JIJEL

TRAVAUX PRATIQUES ESE 23

SYSTEMES EMBARQUES 1ère PARTIE/TP02

Objectifs du TP

1. To know how to make a delay loop with a certain value.

2. To realize how PIC16F84A microcontroller deals with timing issues.

3. To get familiar with interfacing 7-Seg display to PIC16F84A and make a counter.

4. To get familiar with interfacing LED display to PIC16F84A and make a flasher..

5.To know what is the interrupt, and how many sources of interrupt does the PIC16f84A microcontroller have.

6. To know how to make interrupt on PORTB through (RB4 – RB7).

7. To know how to change the flashing speed of this flasher using interrupts method.

8. To know what is the free run timer TMR0; and the function of this timer.

9. To know how to make a counter using TMR0.

Part 1: Flasher (Clignotant à LED)

• Write an assembly program to make a Flasher on RB0 (Hint: Use the Delay

Subroutine).

• Simulate the program using the circuit shown in figure via Proteus software.

Verify it operates properly when simulated

• Program a PIC 16F84A using the Pic  programmer.

• Build the circuit using the programmed PIC 16F84A and then observe its

operation. Demonstrate the circuits operation to the instructor.

Part 2: Counter

• Write an assembly program to make a Counter (0 – 9) on Port A.  Use a pushbutton on RB0  to increment the count(Hint:Use the Delay Subroutine).

• Simulate the program using the circuit shown in figure via Proteus software.

Verify it operates properly when simulated

• Program a PIC 16F84A using the PIC  programmer.

• Build the circuit using the programmed PIC 16F84A and then observe its

• operation. Demonstrate the circuits operation to the instructor.

Part 3:Interruptions

• Write an assembly program to make a Flasher on RA0; then use the interrupt

method to control the flashing speed of this flasher.

Hint// Use the interrupt on change (RB4 – RB7)

• Simulate the program using the circuit shown in figure using  Proteus software.

Verify it operates properly when simulated

• Program a PIC 16F84A using the  pic programmer.

• Build the circuit using the programmed PIC 16F84A and then observe its

operation. Demonstrate the circuits operation to the instructor.

Part4: TMR0

• Write an assembly program to make a Counter  using TMR0; the counter

should increment its value on every 2 pushbuttons on RA4.

• Simulate the program using the circuit shown in figure via Proteus software.

Verify it operates properly when simulated

• Program a PIC 16F84A using the PIC programmer.

• Build the circuit using the programmed PIC 16F84A and then observe its

operation. Demonstrate the circuits operation to the instructor.

Part5: Watchdog Timer (WDT)

• Write an assembly program to make a Counter using WDT; the counter

should increment its value on every one single pushbutton on RA4, and also

counts from 0 to 99.

• Simulate the program using the circuit shown in figure using  Proteus software.

Verify it operates properly when simulated

• Program a PIC 16F84A using the PIC programmer.

Build the circuit using the programmed PIC 16F84A and then observe its

operation. Demonstrate the circuits operation to the instructor

Last Part

• Present your results in a lab report including a copy of the source codes.